Explanation Doc about the SMA to Samtech connect board

Jacky Li on Aug 9th, 2022

Overview:

Front View

There’s a problem with PW1 being the orientation is wrong. If you try to put the module on, you will find the plug is facing inward to the board. Therefore, we jumped it and just use the battery for powering. (The specific of jump can be seen in the back view.)

YOU MUST either have the PW1 module in place, or jump it for it to form a close circuit.

The bits on the switch go from right to left, so if you want 1-0-0 (as the note on the board says) to get Ch0, the correct setting on the switch is actually 0-0-1 (from left to right). This is a mistake in the note on the board.

Update from Jin’s note: “

First, the bits, which you operate switches accordingly, are in reverse order. For example, 001, instead of 100, corresponds to Ch.0. By the way, 0 is when you press the OPEN side of the switch.

Second, for each switch, the channels are counted from the left side of the board, when you are holding the board so that you can read the instructions. This is perfectly logical when it is plugged in an ACDC board (except the switch with a label "ch 0-5" is instead directed to Ch. 24-29, etc.).

On the other hand, when it is plugged in an Anode board (which has its connector in an opposite orientation), the switch with a label "ch 0-5" is indeed directed to Ch. 0-5, but you have to count channels in reverse order for each switch. For example, 001 on the switch with a label "ch 0-5" leads to Ch.5.

”

Back View (Female Samtech connector for the anode)

If you are holding a board with a male Samtech connector, then it is for putting signals into ACDC.

In this view the channels are form left to right 0 -> 29

There’s a problem with this board, being that the Samtech connector was designed for male connectors to input signal into the ACDC.

So if you are using this board for sending signals into ACDC, then it functions as expected.

HOWEVER,

If you are using this board for testing anode, then you need a female Samtech connector, and you need to be extra careful of where each channel should be placed!! The Male to Female is not symmetric because on one side the channel start on pin 10 and the other side pin 11.

The two boards I modified were shifted by one channel, so you only get 29 channels (form 1 to 29), the 0th channel is connected to the ground. (Like this one shown in the back view picture above)

(To fix this you need to somehow re-solder the female connector and move it, in the orientation of the picture shown above, to the left by 2 pins)

For your convenience, I am attaching the designs on the Anode:

Front view of the anode’s Male Samtech connector configuration

This is the same thing on the SMA to Samtech board as well.

Remember, the connection from male to female Samtech means a reverse in left-right. Therefore, your female Samtech should be connected like this following setting:

View of how the Female Samtech connector should fit.

In terms of Soldering: From left to right, if you are soldering a

- Male Samtech connector to the board, make sure the 10th pin on the connector is connected to the first channel of the board.

- Female Samtech connector to the board, make sure the 11th pin on the connector is connected to the first channel of the board.