Fabrication notes for pad_64_in (Square Anodes)

Designer contact: Jacky Li, jackymengyangli@uchicago.edu

Board stack info: 4-layer board, 1.6 mm total thickness, 1/1 oz cu

L1 and L3 are signal planes,

L2 and L4 are ground planes.

We want the impedance of ALL tracing to be 50 ohms.

Using PCB Impedance Calculator | Online Gerber Viewer — GerberViewer.IO,

On L1 (or F.cu in gerber): with model microstrip, T = 0.035 mm, W = 0.2 mm, H = 0.11mm, epsilon = 4.29, we get 48.38 ohms

On L3 (or ln2.cu in gerber): with model asymmetric stripline, T = 0.035mm, W = 0.55 mm, h1 = 0.11mm, h2 = 1.2mm, epsilon = 4.29, we get 50.07 ohms

Other notes: for best performance, fill all vias