Fabrication notes for pad_64_in (Square Anodes)

Designer contact: Jacky Li, jackymengyangli@uchicago.edu

Board stack info: 6-layer board, (customized) total thickness, 1/1 oz cu

2.5 mm, dielectric = 4.29

2.255 mm, dielectric = 4.29

0.11 mm, dielectric = 4.29

1oz cu, 0.035mm

In other words: The layers are organized as

- 0.035mm of Cu   ---------------------------------------------- Signal (L1)

- 0.11 mm of dielectric 4.29 material

- 0.035 mm of Cu   -----------------------Ground (L2)

- 2.255 mm of dielectric 4.29 material

- 0.035 mm of Cu   -----------------------Ground (L3)

- 2.5 mm of dielectric 4.29 material

- 0.035 mm of Cu   --------------------------------------------- Signal (L4)

- 2.5 mm of dielectric 4.29 material

- 0.035 mm of Cu   -----------------------Ground (L5)

- 0.11 mm of dielectric 4.29 material

- 0.035mm of Cu    -----------------------Ground (L6)

L1 and L4 are signal planes,

L2, L3, L5, L6 are ground planes.

We want the impedance of ALL tracing to be 50 ohms.

Using PCB Impedance Calculator | Online Gerber Viewer — GerberViewer.IO,

On L1 (or F.cu in gerber):

- The small traces connects to the SamTech connector is calculated with model microstrip, T = 0.035 mm, W = 0.2 mm, H = 0.11mm, epsilon = 4.29, we get 48.38 ohms

- The striplines are calculated with model Edge-coupled microstrip, T = 0.035mm, W = 4.62mm, S = 2.29mm, h = 2.4mm, epsilon = 4.29, we get 51.82 ohms (odd) and 48.29 ohms (even)

On L4 (or ln3.cu in gerber): with model asymmetric stripline, T = 0.035mm, W = 2.286 mm, h1 = 2.5mm, h2 = 2.5mm, epsilon = 4.29, we get 49.77 ohms

Other notes: for best performance, fill all vias